（Marine Engineering）

轮机专业测试题

注：考虑到面试测评的形式不明了，建议所有题目的答案要熟读。

针对轮机工程技术专业，考察基础机械知识。

(   ) 1. A four-stroke diesel engine needs _______ crankshaft revolutions to complete one cycle.

A. 1

B. 2

C. 3

D. 4

Answer: B. 2

解析： 四冲程机一个工作循环曲轴转两圈。

(   ) 2. The turbocharger helps the engine by _______ .

A. cooling the oil

B. supplying more air into the cylinders

C. filtering fuel

D. heating the water

Answer: B. supplying more air into the cylinders

解析： 废气涡轮增压器把更多空气压入气缸。

(   ) 3. The component that sprays high-pressure fuel into the cylinder is the _______.

A. fuel pump

B. fuel injector

C. turbocharger

D. camshaft

Answer: B. fuel injector

解析： 喷油器（injector）将高压燃油雾化喷入气缸。

(   ) 4. An oily water separator is used to _______.

A. make drinking water

B. remove oil from bilge water before discharge

C. heat the fuel

D. mix oil and water

Answer: B. remove oil from bilge water before discharge

解析： 油水分离器将舱底水中的油分离，使排水含油量低于15ppm。

(   ) 5. Engine jacket water is often cooled by _______ in the heat exchanger.

A. oil

B. fresh water

C. sea water

D. compressed air

Answer: C. sea water

解析： 海水冷却缸套淡水，是常见的间接冷却方式。

(   ) 6. A gear pump is a type of _______ pump.

A. centrifugal

B. positive displacement

C. axial flow

D. jet

Answer: B. positive displacement

解析： 齿轮泵属于容积式泵。

(   ) 7. The governor on a diesel engine controls the _______.

A. temperature

B. engine speed

C. oil pressure

D. air flow

Answer: B. engine speed

解析： 调速器自动控制发动机转速。

(   ) 8. In the refrigeration cycle, the _______ removes heat from the cold room.

A. compressor

B. condenser

C. evaporator

D. expansion valve

Answer: C. evaporator

解析： 蒸发器吸收热量，实现制冷。

(   ) 9. RPM measures _______ .

A. pressure

B. temperature

C. rotational speed

D. fuel consumption

Answer: C. rotational speed

解析： RPM（转/分钟）是转速单位。

(   ) 10. Before maintenance on a pump, you must _______.

A. clean the outside

B. switch off the power and lock it out

C. run the pump at full speed

D. paint it

Answer: B. switch off the power and lock it out

解析： 检修前必须断电并上锁挂牌（Lock-out/Tag-out）。

(   ) 11. A safety valve on a boiler opens to _______ .

A. measure water level

B. release excess pressure

C. add water

D. light the burner

Answer: B. release excess pressure

解析： 安全阀在压力超标时自动开启释放压力。

(   ) 12. In the engine room, you must wear _______ as mandatory PPE.

A. flip-flops

B. safety shoes and helmet

C. sunglasses

D. casual gloves

Answer: B. safety shoes and helmet

解析： 安全鞋、安全帽是机舱的强制个人防护装备。

(   ) 13. The common unit of pressure on board is _______ .

A. kW

B. Bar

C. RPM

D. Volt

Answer: B. Bar

解析： Bar是船上常用的压力单位。

(   ) 14. The oil that reduces friction between moving parts is _______ .

A. fuel oil

B. lubricating oil

C. diesel oil

D. hydraulic oil

Answer: B. lubricating oil

解析： 润滑油（lube oil）用来减少运动部件的摩擦。

(   ) 15. If you find a fuel pipe leaking while the engine is running, you should _______.

A. ignore it until stop

B. report it to the duty engineer immediately

C. repair it yourself quickly

D. clean the floor only

Answer: B. report it to the duty engineer immediately

解析： 发现泄漏必须立即报告值班轮机员，安全第一。